### Inventor 9 MSD Partner Training Event

### Positional Representations Exercise 1

Exercise

Exercise

Positional Representation - Basics

In this exercise you will create a positional representation. You learn how to override assembly constraints in order to configure positional representations within an assembly.

|  | Exercise Steps |
| --- | --- |
| 1. | Open hydro_cylinder.iam from the Brokk Arm System\Assemblies folder. |
| 2. | In the graphics window, drag the Cylinder Piston Rod component. Note: The shaft extension has an open degree of freedom allowing it to be moved back and forth. There are no rotational degrees of freedom. |
| 3. | In the browser, expand the Representations node. |
| 4. | In the browser, right-click the Position node and select New from the menu. Note: You can also create a new positional representation by switching to the Representation browser and selecting the New Positional Representation tool. |
| 5. | Rename the new positional representation to Closed. |
| 6. | In the browser, expand the Cylinder Tube:1 subassembly. Note: There is a suppressed Mate constraint named Drive Stroke. You will use values for this constraint to create a view representation. |
| 7. | In the browser, right-click on the Drive Stroke constraint, and select Override... from the menu. Note: The Override tool is used to modify the solve state of a positional representation. For this positional representation two things need to happen; the first is that the constraint needs to be Enabled, and the second is an offset value needs to be established. |
| 8. | In the Configure Object dialog, on the Constraint tab, click the Override checkbox in the Suppression area, and select Enable from the menu. |
| 9. | In the Configure Object dialog, click the Override checkbox in the Value area, and set the value to 10 mm. |
| 10. | In the Configure Object dialog, click OK. Note: The Drive Stroke constraint has been overridden. In the browser, the text is bold and the cylinder has the shaft pulled in. |
| 11. | In the browser, switch between the Master and Closed positional representations by double-clicking each one. Note: The assembly updates to reflect the active positional representation. |
| 12. | In the browser, switch from the Model browser to the Representations browser. Note: The Representations browser allows close inspection of all Positional Representations associated with the assembly. It shows all overrides for each Positional Representation. |
| 13. | End of exercise. Save the file. You will use this file for the next exercise. |

Positional Representations - Copying & Excel Edit

In this exercise, you will copy existing positional representations and then modify them to create additional positional representations.

|  | Exercise Steps |
| --- | --- |
| 1. | Open hydro_cylinder.iam Note: You must complete the previous exercise before performing the following steps. |
| 2. | In the Browser, activate the Representations browser. |
| 3. | Expand the Positional Representations node. |
| 4. | In the browser, right-click the Closed positional representation, and select Copy from the menu. Note: A new positional representation is made with the same name enumerated. |
| 5. | In the browser, expand both positional representations. Note: All overrides are copied into the new positional representation. |
| 6. | Create another copy of the Closed positional representation and expand all of them in the browser. |
| 7. | Rename the Closed1 positional representation to Middle. Note: To rename a node, single-click to select the node, then single-click a second time to enter the rename mode. Double-clicking will activate the positional representation. |
| 8. | Rename the Closed2 positional representation to Open. |
| 9. | In the Representations browser, click the Edit positional representation table tool. Note: The table is loaded in Microsoft Excel and a table similar to editing an iPart is presented. The master and all positional representations are displayed as rows, and each offset is shown as a different column. This is not an embedded sheet. Tip: You could also edit the offset values for the positional representations by double-clicking their overrides in the browser. |
| 10. | Modify the Middle offset override to 150 mm, and the Open offset override to 330 mm. |
| 11. | From the Excel File menu, click Close & Return to hydro_cylinder.iam. Note: The offset values for the three positional representations are adjusted. |
| 12. | Activate each of the positional representations by double-clicking on them in the browser. Each one shows a different solution state. |
| 13. | End of exercise. Save the file. You will use this file for the next exercise. |

Positional Representations - Documentation

In this exercise, you will create a drawing view and place views of the assembly utilizing different positional representations.

|  | Exercise Steps |
| --- | --- |
| 1. | Open hydro_cylinder.iam Note: You must complete the previous exercise before following these steps. |
| 2. | Create a new drawing. |
| 3. | On the Drawing Views panel bar, click the Base View... tool. |
| 4. | In the Drawing View dialog, select hydro_cylinder from the File drop list, select Left from the Orientation list, and set the scale to 0.25. |
| 5. | In the Drawing View dialog, click the Options tab. |
| 6. | Select the Open positional representation from the Positional Representation list. |
| 7. | Select a location on the drawing sheet to place the view. |
| 8. | On the Drawing Annotation panel bar, select the General Dimension tool. |
| 9. | Add a dimension annotation from the center of the ends of the assembly. |
| 10. | Repeat steps 3 through 9, this time choosing the Closed positional representation. Note: Each view can reference a unique positional representation. |
| 11. | End of exercise. Close the files without saving changes. |