Technical Documentation

3D Part Content Parametric Script Development

Last saved: 12/20/2010 5:40:00 PM Author: Christian Tillich

# Table of Contents

3D Part Content Parametric Construction Development	1

Table of Contents	2

1	Administration	4

1.1	Document History	4

1.2	Team Members	4

1.3	Related Documents	4

2	Introduction	5

2.1	Terminology	5

3	Parametric Construction Scripts (Variant)	6

3.1	Primitives	8

3.1.1	ARC3D	10

3.1.2	ARC3D2	11

3.1.3	ARC3DS	12

3.1.4	BOX	13

3.1.5	CONE	14

3.1.6	CYLINDER	15

3.1.7	ELLIPSOIDHEAD	16

3.1.8	ELLIPSOIDHEAD2	17

3.1.9	ELLIPSOIDSEGMENT	18

3.1.10	HALFSPHERE	19

3.1.11	PYRAMID	20

3.1.12	ROUNDRECT	21

3.1.13	SPHERESEGMENT	22

3.1.14	TORISPHERICHEAD	23

3.1.15	TORISPHERICHEAD2	24

3.1.16	TORISPHERICHEADH	25

3.1.17	TORUS	26

3.2	Member Functions	27

3.2.1	.uniteWith	29

3.2.2	.subtractFrom	30

3.2.3	.intersectWith	31

3.2.4	.erase	32

3.2.5	.rotateX	33

3.2.6	.rotateY	34

3.2.7	.rotateZ	35

3.2.8	.translate	36

3.2.9	.setTransformationMatrix	37

3.2.10	.setPoint	38

3.2.11	.parameters	39

3.2.12	.transformationMatrix	40

3.2.13	.numberOfPoints	41

3.2.14	.pointAt	42

3.2.15	.directionAt	43

3.3	Functions	44

3.3.1	activate	45

3.3.1.1	dbtype	46

3.3.2	TESTACPSCRIPT / TESTACPSCRIPT1	48

3.3.3	demand loader	49

4	variants.zip	50

4.1	amake.bat	51

5	Anything else	53

# Administration

## Document History

| Date | Name | Status | Comments |
| --- | --- | --- | --- |
| 18 Oct. 2008 | Christian Tillich | Preliminary draft | Initial pass |
| Date Revised |  | Enter status |  |

## Team Members

| Role | Name | Date of Review and Sign-off |
| --- | --- | --- |
| Development Lead | Christian Tillich | Date of Review |
| Product Management |  | Date of Review |
| Project Management |  | Date of Review |
| SWD Management |  | Date of Review |
| TD |  | Date of Review |
| TD Management |  | Date of Review |
| User Assistance |  | Date of Review |

## Related Documents

| File Name | Description |
| --- | --- |

# Introduction

This document describes the Autodesk Internal ‘Content Parametric Script Design Process’ for generating Plant 3D parts.

This document outlines the design of parametric construction scripts and covers the detailed steps to complete a Content Build.

The scope of this document is limited to 3D Part Content, as it will be published in a Part Catalog database.

## Terminology

| Term | Definition |
| --- | --- |
| Catalogs | A repository of parts which are referenced into the 3D model for various reasons. The parts are organized in a general fashion so that a user can browse through them for their various applications. |
| Variant | Synonym for parametric construction script |

# Parametric Construction Scripts (Variant)

Plant 3D is delivered with a large catalog of predefined parts. To get 3D representations of these parts into 3D drawings "variant programs" are used. A "variant" is a small (Python) subroutine that takes the dimensions of a specific part as input and creates a 3D representation (typically a solid within a block) as output.

Approximately 20.000 different variant subroutines are part of the package, and additional ones can easily be added. The available variants cover nearly all types of parts commonly used in plant design: pipes, elbows, flanges, tees, crosses, nozzles, olets, different types of valves and many more.

The variant are created by using default, C predefined, primitives but each variant can also be used by another variant.

The primitives can be positioned, combined, subtracted,… with different member functions.

And we use some functions to setup using the variant inside Plant 3D.

A typical variant looks like this:

def CPFLR(s, L=22.0, D1=220.0, D2=114.3, ID="CPFLR", **kw):

o0=CYLINDER(s, R=D1/2.0, H=L, O=D2/2.0).rotateY(90.0)

s.setPoint((0.0, 0.0, 0.0), (-1.0, 0.0, 0.0), 0.0)

s.setPoint((0.0, 0.0, 0.0), ( 1.0, 0.0, 0.0), 0.0)

This one creates a simple lapped flange with two connection points (ports). Connection points have a position and a direction vector that allows for automatic alignment of connected parts.

To get the data from database we need also the database field information. That is also done inside the variant file at the construction routine (normally direct after the variant definition end) and looks like, for the example above:

activate(CPFLR,

"""

[Type AQA-VCPFLR]

VID=STRING,32

DN=STRING

L=LENGTH

D1=LENGTH

D2=LENGTH

Units=STRING,8

;

uniqId=CALC =$self.VID$ $self.DN$

;

boltCalcVal=CALC CPF_GetNeededBoltParam(B=self.L.value(), L=self.L.value())

;

@key=VID,DN

""",

"@VarDataDefault0",

)

This defines for the lapped flange now the parameter of the parametric construction data that has to be stored inside database or needed for editors, the needed fields and some calculated fields.

The variants can also be tested with the AutoLISP® function TESTACPSCRIPT and/or TESTACPSCRIPT1.

The variants are loaded on demand to save memory. Plant 3D has created his own demand loader for the variants.

The variants are stored after installation in a zip archive and get read from the archive. That saves some time and doesn't fill the hard disk with much directories and files.

## Primitives

Plant 3D offers a lot of Primitives (basic parametric constructions) that are written inside C++ and are the interface to the CAD system.

There are:

- ARC3D
defines a 'normal' elbow

- ARC3D2
defines a reduced elbow

- ARC3DS
defines a segmented elbow

- BOX
defines a box

- CONE
defines a cone and also a frustum

- CYLINDER
defines a 'normal' or an elliptical cylinder

- ELLIPSOIDHEAD
defines a normalized ellipsoid head

- ELLIPSOIDHEAD2
defines an ellipsoid head (0.5 * R)

- ELLIPSOIDSEGMENT
defines an ellipsoid (football/rugby ball like)

- HALFSPHERE
defines a half sphere

- PYRAMID
defines a pyramid or a frustum of pyramid

- ROUNDRECT
defines a transition from a rectangle to a circle

- SPHERESEGMENT
defines a sphere segment

- TORISPHERICHEAD
defines a normalized torispheric head

- TORISPHERICHEAD2
defines a torispheric head (small radius = 25.00)

- TORISPHERICHEADH
defines a normalized torispheric head with height

- TORUS
defines a torus

Each created variant can also be used as primitive while creating other geometry bodies

### ARC3D

ARC3D defines a 'normal' elbow

a call from Python should be like

s=ARC3D(s, D, R, A)

The parameters means:

| s | the main object |
| --- | --- |
| D | the 1/2 diameter |
| R | the bend radius |
| A | the bend angle |

The base point is the intersection between the thought centerline trough booth ends

### ARC3D2

ARC3D2 defines a reduced elbow (also a 'normal' elbow can be done with them)

a call from Python should be like

s=ARC3D2(s, D, D2, R, A)

The parameters means:

| s | the main object |
| --- | --- |
| D | the 1/2 diameter |
| D2 | the second 1/2 diameter - if not set D ist used as D2 |
| R | the bend radius |
| A | the bend angle |

The base point is the intersection between the thought centerline trough booth ends

### ARC3DS

ARC3DS defines a segmented elbow

a call from Python should be like

s=ARC3DS(s, D, D2, R, A, S)

The parameters means:

| s | the main object |
| --- | --- |
| D | the 1/2 diameter |
| R | the bend radius |
| A | the bend angle |
| S | number of segments |

The base point is the intersection between the thought centerline trough booth ends

### BOX

BOX defines a box

a call from Python should be like

s=BOX(s, L, W, H)

The parameters means:

| s | the main object |
| --- | --- |
| L | the box length |
| W | the box width |
| H | the box height |

The base point is the center of gravity

### CONE

CONE defines a cone and also a frustum

a call from Python should be like

s=CONE(s, R1, R2, H, E)

The parameters means:

| s | the main object |
| --- | --- |
| R1 | the bottom radius |
| R2 | the upper radius - if 0.0 a full cone, if > 0.0 a frustum is created |
| H | the height |
| E | the eccentricity between upper and bottom center |

The base point is the center of bottom

### CYLINDER

CYLINDER defines a 'normal' or an elliptical cylinder

a call from Python should be like

s=CYLINDER(s, R, H, O)

or

s=CYLINDER(s, R1, R2, H, O)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |
| R1 | the 1. radius |
| R2 | the 2. radius |
| H | the height |
| O | the hole radius |

The base point is the center of bottom

### ELLIPSOIDHEAD

ELLIPSOIDHEAD defines a normalized ellipsoid head

a call from Python should be like

s=ELLIPSOIDHEAD(s, R)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |

The base point is the center of bottom

with obj.parameters() we can get the height (H) of the primitive

Calculates the intersection iteratively. ELLIPSOIDHEAD is formed as an ellipsis. The body is interpolated with cones which are used to find the intersection points.

### ELLIPSOIDHEAD2

ELLIPSOIDHEAD2 defines a ellipsoid head (0.5 * R)

a call from Python should be like

s=ELLIPSOIDHEAD2(s, R)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |

The base point is the center of bottom

with obj.parameters() we can get the height (H) of the primitive

Calculates the intersection iteratively. ELLIPSOIDHEAD2 is formed as an ellipsis with the main axes a = radius and b = 0.5*radius .The body is interpolated with cones which are used to find the intersection points.

### ELLIPSOIDSEGMENT

ELLIPSOIDSEGMENT defines an ellipsoid body (like a football/rugby ball)

a call from Python should be like

s=ELLIPSOIDSEGMENT(s, RX, RY, A1, A2, A3, A4)

The parameters means:

| s | the main object |
| --- | --- |
| RX | big ellipsoid axis |
| RY | small ellipsoid axis |
| A1 | complete rotation angle |
| A2 | start angle of rotation |
| A3 | start angle of ellipse |
| A4 | end angle of ellipse |

The base point is the center of gravity

### HALFSPHERE

HALFSPHERE defines a half sphere

a call from Python should be like

s=HALFSPHERE(s, R)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |

The base point is the center of bottom

### PYRAMID

PYRAMID defines a pyramid or a frustum of pyramid

a call from Python should be like

s=PYRAMID(s, L, W, H, HT)

The parameters means:

| s | the main object |
| --- | --- |
| L | the length |
| W | the width |
| H | the frustum height if HT isn't set it's the total pyramid height |
| HT | the total pyramid height |

The base point is the center of the bottom rectangle

### ROUNDRECT

ROUNDRECT defines a transition from a rectangle to a circle

a call from Python should be like

s=ROUNDRECT(s, L, W, H, R2, E)

The parameters means:

| s | the main object |
| --- | --- |
| L | the length |
| W | the width |
| H | the height |
| R2 | the circle radius |
| E | the eccentricity between upper and bottom center |

The base point is the center of the rectangle

### SPHERESEGMENT

SPHERESEGMENT defines a sphere segment

a call from Python should be like

s=SPHERESEGMENT(s, R, P, Q)

The parameters means:

| s | the main object |
| --- | --- |
| R | the sphere radius |
| P | the segment height |
| Q | the height where the segment starts from center |

The base point is the center of the lower part of the segment.

### TORISPHERICHEAD

TORISPHERICHEAD defines a normalized torispheric head

a call from Python should be like

s=TORISPERICHEAD(s, R)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |

The base point is the center of bottom

with obj.parameters() we can get the height (H) of the primitive.

### TORISPHERICHEAD2

TORISPHERICHEAD2 defines a torispheric head (small radius = 25.00)

a call from Python should be like

s=TORISPERICHEAD2(s, R)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |

The base point is the center of bottom

with obj.parameters() we can get the height (H) of the primitive.

### TORISPHERICHEADH

TORISPHERICHEADH defines a normalized torispheric head with height

a call from Python should be like

s=TORISPERICHEADH(s, R, H)

The parameters means:

| s | the main object |
| --- | --- |
| R | the radius |
| H | the height |

The base point is the center of bottom.

### TORUS

TORUS defines a torus

a call from Python should be like

s=TORUS(s, R1, R2)

The parameters means:

| s | the main object |
| --- | --- |
| R1 | the outer radius |
| R2 | the inner radius |

The base point is the intersection between the thought centerline trough booth ends.

## Member Functions

Plant 3D offers some member functions to modify objects (primitives) like rotate, move,...

There are also some member request functions to get additional information from the object.

- Modifier functions

  - obj.uniteWith
unites 2 objects

  - obj.subtractFrom
subtracts 1 object from another 1

  - obj.intersectWith
creates an intersection of 2 objects

  - obj.erase
removes an object from memory

  - obj.rotateX
rotate the object round the X-axis

  - obj.rotateY
rotate the object round the Y-axis

  - obj.rotateZ
rotate the object round the Z-axis

  - obj.translate
moves the object

  - obj.setTransformationMatrix
set the object's transformation matrix

  - obj.setPoint
append connection point to the body

- Request functions

  - obj.parameters
return the object's construction parameters

  - obj.transformationMatrix
return the object's current transformation matrix

  - obj.numberOfPoints
return number of (connection) points

  - obj.pointAt
return the position of a connection point

  - obj.directionAt
return the direction at a connection point

### .uniteWith

The modifier function .uniteWith unites the calling object obj with the second object oobj.

The second object oobj is given to the function as argument.

The result is set to obj.

After the unite the second object oobj has to be removed from memory with .erase

a call from Python should be like

obj.uniteWith(oobj)

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0) ).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0))

### .subtractFrom

The modifier function .subtractFrom subtracts the second object oobj from the calling object obj.

The second object oobj is given to the function as argument.

The result is set to obj.

After the subtract the second object oobj has to be removed from memory with .erase.

a call from Python should be like

obj.subtractFrom(oobj)

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0))

### .intersectWith

The modifier function .intersectWith creates an intersect object with the calling object obj and the second object oobj.

The second object oobj is given to the function as argument.

The result is set to obj.

After the intersect the second object oobj has to be removed from memory with .erase.

a call from Python should be like

obj.intersectWith(oobj)

It looks like:

def CADAPT_SQ2RO_Sub(s, LL=715.0, LW=700.0, R=225.0, OL=919.19, OW=698.28, H=2515.45, ID="CADAPT_SQ2RO_Sub", **kw):

TL=R*2.0

TW=R*2.0

RB=sqrt(pow(LL, 2.0)+pow(LW, 2.0))/2.0

RA=90.0-asDegrees(atan2(OL, OW))

RE=sqrt(pow(OL, 2.0) + pow(OW, 2.0))

o1=CONE(s, R1=RB, R2=R, H=H, E=RE).rotateZ(RA)

o2=CADAPT_SQ2SQ_Sub(s, LL=LL, LW=LW, TL=TL, TW=TW, OL=OL, OW=OW, H=H)

o1.intersectWith(o2)

o2.erase()

s.setPoint((0.0, 0.0, 0.0), (0.0, 0.0, -1.0))

s.setPoint((OL,  OW,  H  ), (0.0, 0.0,  1.0))

### .erase

The modifier function .erase has always to be called after .uniteWith, .subtractFrom and .intersectWith.

.erase removes obj from the memory.

a call from Python should be like

obj.erase()

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0) ).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0))

### .rotateX

The modifier function .rotateX rotates the calling object obj around the X-axis.

The rotation angle a has to be given as degree.

a call from Python should be like

obj.rotateX(a)

It looks like:

def CSGC004(s, D=116.0, L=30.0, W=4.0, K=0.0, H=95.0, M1=10.0, M2=0.0, ID="CSGC004", **kw):

R1=D/2.0

R2=R1+W

H1=H-M2

if H1<(R2+W) or M2==0.0:

H1=R2+W

KWdt=M1*tan(aqa.math.asRadiants(30.0))

H2=H-H1

o1a=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0))

o1b=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0)).rotateX( 60.0)

o1c=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0)).rotateX(-60.0)

o2=CSGC003(s, D=D, L=L, W=W, K=K, H=H1-R1)

o1a.uniteWith(o1b)

o1b.erase()

o1a.uniteWith(o1c)

o1c.erase()

o1a.rotateZ(180.0)

o1a.uniteWith(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((0, 0, 0), (1, 0, 0))

s.setPoint((H, 0, 0), (1, 0, 0))

### .rotateY

The modifier function .rotateY rotates the calling object obj around the Y-axis.

The rotation angle a has to be given as degree.

a call from Python should be like

obj.rotateY(a)

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0))

### .rotateZ

The modifier function .rotateZ rotates the calling object obj around the Z-axis.

The rotation angle a has to be given as degree.

a call from Python should be like

obj.rotateZ(a)

It looks like:

def CSGC004(s, D=116.0, L=30.0, W=4.0, K=0.0, H=95.0, M1=10.0, M2=0.0, ID="CSGC004", **kw):

R1=D/2.0

R2=R1+W

H1=H-M2

if H1<(R2+W) or M2==0.0:

H1=R2+W

KWdt=M1*tan(aqa.math.asRadiants(30.0))

H2=H-H1

o1a=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0))

o1b=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0)).rotateX( 60.0)

o1c=BOX(s, L=M1, W=KWdt, H=H2).translate((0.0-(H2/2.0)-H1, 0.0, 0.0)).rotateX(-60.0)

o2=CSGC003(s, D=D, L=L, W=W, K=K, H=H1-R1)

o1a.uniteWith(o1b)

o1b.erase()

o1a.uniteWith(o1c)

o1c.erase()

o1a.rotateZ(180.0)

o1a.uniteWith(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((0, 0, 0), (1, 0, 0))

s.setPoint((H, 0, 0), (1, 0, 0))

### .translate

The modifier function .translate moves the calling object obj along the given vector v.

The argument v can be a mPoint, mVector or a 3-Tupel (x, y, z).

a call from Python should be like

obj.translate(v)

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0) ).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0))

### .setTransformationMatrix

The modifier function .setTransformationMatrix sets the object's transformation matrix t.

a call from Python should be like

obj.setTransformationMatrix(t)

It looks like:

def CPNO(s, D=114.3, R=300.0, L=100.0, D2=0.0, A=90.0, OF=0.0, ID="CPNO", **kw):

vC=CPNO_util(D=D, D2=D2, OF=OF, L=L, R=R, A=A)

R=vC[0]/2.0

R2=vC[1]/2.0

o1=ARC3D(s, D=R, R=vC[6], A=vC[2])

p2 = o1.pointAt(1)

if vC[2] == 90.0:

xMove = 0.0 - p2.x()

yMove = vC[7] - p2.y()

else:

xMove = 0.0 - p2.x() - (L * cos(asRadiants(vC[2])))

yMove = vC[7] - p2.y() - (L - (L * sin(asRadiants(vC[2]))))

t1 = mTransform().translate((xMove, yMove, 0.0))

o1.setTransformationMatrix(t1)

s.setPoint((0, 0, 0), (0, -1, 0))

s.setPoint(t1 * o1.pointAt(1), o1.directionAt(1))

if vC[5]:

d1 = ARC3D(s, D=R-vC[4], R=vC[6], A=vC[2])

d1.setTransformationMatrix(t1)

o1.subtractFrom(d1)

d1.erase()

o3=CYLINDER(s, R=R2, H=vC[6]*2.0, O=0.0).rotateY(270).translate((vC[6], 0, 0))

if vC[7] < vC[6]:

o2 = BOX(s, H=vC[6]*2.0, W=vC[0]*2.0, L=vC[6]).translate(((vC[6]/2.0), -(vC[6]/2.0), 0.0))

o3.uniteWith(o2)

o2.erase()

o1.subtractFrom(o3)

o3.erase()

### .setPoint

The modifier function .setPoint  insert a point at position p and set the direction v to the next part inside this point.

Alternative we can also set a rotation angle a (most needed for elliptical flanges to get the right alignment)

The argument p and v can be a mPoint, mVector or a 3-Tupel (x, y, z).

The rotation angle a has to be given as degree.

a call from Python should be like

obj.setPoint(p,v)

or

obj.setPoint(p,v,a)

It looks like:

def CPMB(s, L=54.0, B=22.0, D1=220.0, D2=114.3, ID="CPMB", **kw):

O=CON_OF_DIV(D2)/2.0

o1=CYLINDER(s, R=D2/2.0, H=L-B, O=O).rotateY(90).translate((B, 0, 0))

o0=CYLINDER(s, R=D1/2.0, H=B, O=O).rotateY(90)

o0.uniteWith(o1)

o1.erase()

o2=CYLINDER(s, R=O, H=L-B, O=0.0) ).rotateY(90).translate((B, 0, 0))

o0.subtractFrom(o2)

o2.erase()

s.setPoint((0, 0, 0), (-1, 0, 0))

s.setPoint((L, 0, 0), ( 1, 0, 0), 0)

### .parameters

The request function .parameters returns the object's construction parameters

a call from Python should be like

obj.parameters()

It looks like:

def CPC(s, D=114.30, L=64.00, DISP=0, OF=-1.0, ID="CPC", **kw):

if DISP == 0:

o2 = TORISPHERICHEAD(s, R=D/2.0).rotateY(90.0)

vT = o2.parameters()

vTH = float(vT["H"])

if L - vTH < 0.0:

o2.erase()

o2 = TORISPHERICHEAD(s, R=D/2.0, H=L).rotateY(90.0)

vTH = L

o2.translate((L-vTH, 0.0, 0.0))

else:

vTH = CON_OF_MULT(D)-D

o2 = CYLINDER(s, R=(D+vTH)/2.0, H=vTH, O=0.0).rotateY(90).translate((L-vTH, 0.0, 0.0))

if L - vTH > 0.0:

o1 = CYLINDER(s, R=D/2.0, H=L-vTH, O=0.0).rotateY(90.0)

o1.uniteWith(o2)

o2.erase()

if not OF == 0.0:

if OF == -1:

O = CON_OF_DIV(D)

else:

O = D-(OF * 2.0)

L2 = L-(D-O)+OF

if DISP == 0:

o4 = TORISPHERICHEAD(s, R=O/2.0).rotateY(90.0)

o4.translate((L-vTH, 0.0, 0.0))

else:

o4 = None

if L - vTH > 0.0:

o3 = CYLINDER(s, R=O/2.0, H=L-vTH, O=0.0).rotateY(90.0)

if o4:

o3.uniteWith(o4)

o4.erase()

else:

o3 = o4

o1.subtractFrom(o3)

o3.erase()

s.setPoint((0.0, 0.0, 0.0), (-1.0, 0.0, 0.0), 0.0)

### .transformationMatrix

The request function .transformationMatrix returns the object's current transformation matrix

a call from Python should be like

obj.transformationMatrix()

(sorry, not used inside any of the existing parametric constructions so I can’t show a real used example)

### .numberOfPoints

The request function .numberOfPoints returns the object's number of (connection) points

a call from Python should be like

obj.numberOfPoints()

(sorry, not used inside any of the existing parametric constructions so I can’t show a real used example)

### .pointAt

The request function .pointAt returns the position of connection point n as mPoint.

If inECS is set to 1 the mPoint is not transformed to WCS - default for inECS is 0.

a call from Python should be like

obj.pointAt(n, inECS)

It looks like:

def CPNO(s, D=114.3, R=300.0, L=100.0, D2=0.0, A=90.0, OF=0.0, ID="CPNO", **kw):

vC=CPNO_util(D=D, D2=D2, OF=OF, L=L, R=R, A=A)

R=vC[0]/2.0

R2=vC[1]/2.0

o1=ARC3D(s, D=R, R=vC[6], A=vC[2])

p2 = o1.pointAt(1)

if vC[2] == 90.0:

xMove = 0.0 - p2.x()

yMove = vC[7] - p2.y()

else:

xMove = 0.0 - p2.x() - (L * cos(asRadiants(vC[2])))

yMove = vC[7] - p2.y() - (L - (L * sin(asRadiants(vC[2]))))

t1 = mTransform().translate((xMove, yMove, 0.0))

o1.setTransformationMatrix(t1)

s.setPoint((0, 0, 0), (0, -1, 0))

s.setPoint(t1 * o1.pointAt(1), o1.directionAt(1))

if vC[5]:

d1 = ARC3D(s, D=R-vC[4], R=vC[6], A=vC[2])

d1.setTransformationMatrix(t1)

o1.subtractFrom(d1)

d1.erase()

o3=CYLINDER(s, R=R2, H=vC[6]*2.0, O=0.0).rotateY(270).translate((vC[6], 0, 0))

if vC[7] < vC[6]:

o2 = BOX(s, H=vC[6]*2.0, W=vC[0]*2.0, L=vC[6]).translate(((vC[6]/2.0), -(vC[6]/2.0), 0.0))

o3.uniteWith(o2)

o2.erase()

o1.subtractFrom(o3)

o3.erase()

### .directionAt

The request function .directionAt returns the direction of connection point n as mVector.

If inECS is set to 1 the mVector is not transformed to WCS - default for inECS is 0.

a call from Python should be like

obj.directionAt(n, inECS)

It looks like:

def CPNO(s, D=114.3, R=300.0, L=100.0, D2=0.0, A=90.0, OF=0.0, ID="CPNO", **kw):

vC=CPNO_util(D=D, D2=D2, OF=OF, L=L, R=R, A=A)

R=vC[0]/2.0

R2=vC[1]/2.0

o1=ARC3D(s, D=R, R=vC[6], A=vC[2])

p2 = o1.pointAt(1)

if vC[2] == 90.0:

xMove = 0.0 - p2.x()

yMove = vC[7] - p2.y()

else:

xMove = 0.0 - p2.x() - (L * cos(asRadiants(vC[2])))

yMove = vC[7] - p2.y() - (L - (L * sin(asRadiants(vC[2]))))

t1 = mTransform().translate((xMove, yMove, 0.0))

o1.setTransformationMatrix(t1)

s.setPoint((0, 0, 0), (0, -1, 0))

s.setPoint(t1 * o1.pointAt(1), o1.directionAt(1))

if vC[5]:

d1 = ARC3D(s, D=R-vC[4], R=vC[6], A=vC[2])

d1.setTransformationMatrix(t1)

o1.subtractFrom(d1)

d1.erase()

o3=CYLINDER(s, R=R2, H=vC[6]*2.0, O=0.0).rotateY(270).translate((vC[6], 0, 0))

if vC[7] < vC[6]:

o2 = BOX(s, H=vC[6]*2.0, W=vC[0]*2.0, L=vC[6]).translate(((vC[6]/2.0), -(vC[6]/2.0), 0.0))

o3.uniteWith(o2)

o2.erase()

o1.subtractFrom(o3)

o3.erase()

## Functions

The variants need and offer some functions to link to Plant 3D or for testing:

There are:

- activate()
activate the variant and defines the needed database fields

- TESTACPSCRIPT
an AutoLISP® function to see and test the variant parameters

- demand loader
Loader for the variants on demand

### activate

To use a variant definition as Plant 3D part we have to activate the function.

To get and convert the data from database we need also the field information. That is also done inside activate.

a call from Python should be like

activate(func, dbtype, defkey)

The parameters means:

| func | the Python function to activate (in this case a variant) |
| --- | --- |
| dbtype | this is the database definition. this definition has to be done between """ and """ (each has 3 quotation marks) |
| defkey | database definition key (it’s always "@VarDataDefault0" inside Plant 3D) |

For example:

activate(CPFLR,

"""

[Type AQA-VCPFLR]

VID=STRING,32

DN=STRING

L=LENGTH

D1=LENGTH

D2=LENGTH

Units=STRING,8

;

uniqId=CALC =$self.VID$ $self.DN$

;

boltCalcVal=CALC CPF_GetNeededBoltParam(B=self.L.value(), L=self.L.value())

;

@key=VID,DN

""",

"@VarDataDefault0",

)

This defines for the lapped flange now the needed fields and some calculated fields.

#### dbtype

This is the database / parameter definition.

This definition has to be done between """ and """ (each has 3 quotation marks)

And then it looks like:

"""

[Type AQA-VCPFLR]

VID=STRING,32

DN=STRING

L=LENGTH

D1=LENGTH

D2=LENGTH

Units=STRING,8

;

uniqId=CALC =$self.VID$ $self.DN$

;

boltCalcVal=CALC CPF_GetNeededBoltParam(B=self.L.value(), L=self.L.value())

;

@key=VID,DN

""",

Its defined as:

- Each dbtype definition starts with 3 quotation marks
“””

- The next line describes the variant name and should start with [AQA_V and end with ]
[Type AQA-VCPFLR]
(the definition with AQA-V comes from the adapter which read ACPlant Designer parametric scripts and can’t changed now)

- The next 2 lines are also always the same (come also from adapter and should be added for compatibility)
VID=STRING,32
DN=STRING

- Now we define for each needed script argument an extra definition line
each line starts with the argument name then we use the = sign as delimiter and now we add the type.
After the definition we can also add char, numbers length , delimited with ‘,’, but not needed inside Plant 3D (ACPlant Designer used this definition to create database tables)
DN=STRING
L=LENGTH
D1=LENGTH
D2=LENGTH
Units=STRING,8

allowed values for the  type are

  - LENGTH		for length values that also can be calculated to another unit (like mm -> in)

  - ANGLE		is used for set and real angle valve

  - INT		a real integer value

  - DOUBLE		a float value

  - STRING		a string

- Now we can define some help fields like
uniqId		(for calculating unique blocknames)
boltCalcVal,…
we can set here each parameter that we want and with =CALC we can call each function that we want and set the return value inside the wished parameter
uniqId=CALC =$self.VID$ $self.DN$
boltCalcVal=CALC CPF_GetNeededBoltParam(B=self.L.value(), L=self.L.value())

- The line @key= was also use inside ACPlant Designer for database creation and defined the table index
@key=VID,DN

- Comments can be set with ‘;’

- And now comes the ending 3 quotation marks
“””

### TESTACPSCRIPT / TESTACPSCRIPT1

To test and check the geometry body we have the AutoLISP® function TESTACPSCRIPT and TESTACPSCRIPT1 created.

The first parameter at booth script types is the variants name (like "CPFLR")

The next parameters for TESTACPSCRIPT are couples of values: the fieldname and the needed value (like "D1" "300.5").

For TESTACPSCRIPT1 we have to set all the parameters inside 1 string (like it’s done inside catalog/spec dbs: “D1=300.5”)

If a field isn't set the function takes the default values from the variants definition.

That looks like:

(TESTACPSCRIPT "CPFLR")

(TESTACPSCRIPT "CPFLR" "D1" "300.5")

(TESTACPSCRIPT "CPFLR" "L" "40" "D1" "300.5" "D2" "88.9")

Or

(TESTACPSCRIPT1 "CPFLR")

(TESTACPSCRIPT1 "CPFLR" "D1=300.5")

(TESTACPSCRIPT1 "CPFLR" "L=40,D1=300.5,D2=88.9")

Before we can use TESTACPSCRIPT or TESTACPSCRIPT1 inside Plant 3D we have to load PnP3dACPAdapter.arx.

That can be done with

(arxload "PnP3dACPAdapter.arx")

or with the appload function

### demand loader

Approximately 20.000 different variant subroutines needs there time to load. To speed up the start procedure we created the Variant Demand Loader.

First we packed all the variants into an zip archive so we solved the operating system problems with the much directories - Python use this zip archive as package and get the files really fast.

At second we load the variants only if we need them. For that we created a dummy loader file that includes only links to the variants.

This file is compiled at build time to variants.map.

The contents of the file dummy_var.load looks like (variant;python_package_like_location):

CPFBO;varmain.flangesub.cpfbo

CPFBR;varmain.flangesub.cpfbr

CPFLO;varmain.flangesub.cpflo

CPFLR;varmain.flangesub.cpflr

CPFWO;varmain.flangesub.cpfwo

CPFWO_F_SF;varmain.flangesub.cpfwo_f_sf

CPFWR;varmain.flangesub.cpfwr

CPFWR_F_SF;varmain.flangesub.cpfwr_f_sf

CPFWRI;varmain.flangesub.cpfwri

CPFWRI_F_SF;varmain.flangesub.cpfwri_f_sf

FLANGE;varmain.flangesub.cpf_old

CPFW;varmain.flangesub.cpf_old

# variants.zip

All variants are organized inside a directory structure.

Normally each variant is defined inside 1 separate file.

Some variant have also sub files.

So we get very much directories and files that make some problems with slow hard disk (it slows down the machine).

To solve that we pack all the directories and variant files inside variant.zip. Python can handle this archive as package and is very fast.

What is all inside variant.zip:

- the compile variant files (*.pyc)

- dimensioned images with resolution 640x640px per script (*_640.png)

- basic thumbnail images per script with resolution

  - 200x200px	(*_200-png)

  - 64x64px	(*_64.png)

  - 32x32px	(*_32.png)

- Script metadata	(*.xlm)

Variant.zip is build with p3dmakeall.bat (or you call amake.bat inside directory “…\plant\Develop\Piping\3rdParty\ACP”

## amake.bat

That Plant 3D can work with all the variant scripts we have to generate some files like variants.zip, variants.map and variants.xlm.

amake.bat helps us to generate this files and looks like (batch content / comment not in batch):

@rem $Header: //depot/plant/Develop/PnID/Source/Components/amake.bat#2 $

@echo off

if (%ACTOP%) == () goto BADACADROOT

…set the base environment  vars…

@set PYTHONPATH=%PIPING_3RDPARTY_ACP%\variants\aqa\varmap;%PIPING_3RDPARTY_PYTHON%\Source\Lib;%PIPING_3RDPARTY_PYTHON_BIN%

@echo pnproot is %PNPROOT%

@echo PYTHON_EXE is %PYTHON_EXE%

@echo PIPING_3RDPARTY_ACP is %PIPING_3RDPARTY_ACP%

@rem -----------------------------------------------------------------------

@if not exist %PIPING_3RDPARTY_ACP%\buildzip.py goto BADBLDROOT

@cd %PIPING_3RDPARTY_ACP%

@set CONTENTTOOLPATH=%PIPING_3RDPARTY_ACP%\ContentTools

@set XLSREADERPATH=%PIPING_3RDPARTY_ACP%\ContentTools\XmlCreator

@if /i "%1" == "clean" goto makeClean

… remove existing error files

@rm -f %CONTENTTOOLPATH%\ContentTools.err

… build xlm creator tools

@devenv /useenv %CONTENTTOOLPATH%\ContentTools.sln /build %BLDTYPE%  /out %CONTENTTOOLPATH%\ContentTools.err

@if exist %CONTENTTOOLPATH%\ContentTools.err typeerr.exe %CONTENTTOOLPATH%\ContentTools.err

%CBIN%\XmlCreator -img %PIPING_CONTENT%\Templates\Piping.dcfx %PIPING_CONTENT%\Templates\PipingClassToImgRel.xml

… compile all the python files, create also variants.map

%PYTHON_EXE% %PIPING_3RDPARTY_ACP%\buildzip.py --build

… build all the needed xlm files

%CBIN%\XmlCreator -acp %PIPING_3RDPARTY_ACP%\variants\variant_parameter_doc.xlsx %PIPING_3RDPARTY_ACP%\dummy_var.load %PIPING_3RDPARTY_ACP%\log.txt %PIPING_3RDPARTY_ACP%\variants.xml

… now create variants.zip

%PYTHON_EXE% %PIPING_3RDPARTY_ACP%\buildzip.py --zip

@goto DONE

:makeClean

@cd %PIPING_3RDPARTY_ACP%

@%CBIN%\XlsToXml.exe -d %PIPING_3RDPARTY_ACP%\log.txt

@devenv /useenv %CONTENTTOOLPATH%\ContentTools.sln /clean %BLDTYPE%

@rm -f %PIPING_3RDPARTY_ACP%\intermediaryXml.xml %PIPING_3RDPARTY_ACP%\scriptInXls.map

%PYTHON_EXE% %PIPING_3RDPARTY_ACP%\buildzip.py --clean

@rm -f %PIPING_3RDPARTY_ACP%\VARIANTS.zip %PIPING_3RDPARTY_ACP%\VARIANTS.map

@goto DONE

@rem -----------------------------------------------------------------------

:BADBLDROOT

@echo The PIPING_3RDPARTY_ACP environment variable has incorrect value

@goto DONE

@rem -----------------------------------------------------------------------

:BADACADROOT

echo The ACTOP environment variable has incorrect value

goto done

:DONE

@set PYTHONPATH=